Active Copper (ACC) Cable

TECHNICAL DATA SHEET

OSFP 400G to OSFP 400G ACC

2.5~5.0M	PVC Black

# Key Features

- Support 8x56G PAM4

- 400G To 400G Data Rate

- 3.3V Power Supply

- Hot Pluggable

- Excellent SI Performance

- Efficient Patching

- RoHS Compliance

# Applications

- Data Center & Networking Equipment

- Servers/Storage Devices

- High Performance Computing (HPC)

- Switches/Routers

- Telecom Central Offices

# Supported Standards

- OSFP MSA Rev 4.1

- IEEE 802.3cd

- IEEE 802.3bj

- I2C for EEPROM

- CMIS 5.0

# Ordering Information

| LFOO-PK1xxxC | OSFP 400G to OSFP 400G ACC | — 2.5~5.0M |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ordering P/Ns | Description | Length | AWG | Cable | NOTE |
| LFOO-PK1x25C | OSFP 400G to OSFP 400G ACC | 2.5m | 30 AWG | PVC Black |  |
| LFOO-PK1x35C | OSFP 400G to OSFP 400G ACC | 3.5m | 28 AWG | PVC Black |  |
| LFOO-PK1x50C | OSFP 400G to OSFP 400G ACC | 5.0m | 26 AWG | PVC Black |  |
| xxx define ACC cable length |  |  |  |  |  |

1

www.lumulus.com  9685 Via Excelencia, San Diego, CA 92126	Rev.A—14/04/2026

Images are for illustrative purposes only and may not accurately represent the actual product. Specifications and appearances are subject to change without prior notice.

# Description

The LFOO-PK1xxxC, OSFP 400G Active Redriver Copper Cable Assembly, meticulously designed for application in the tele-communications and data center sectors. It facilitates bidirectional transmission of 400Gb traffic per cable, accommoda-ting 8 lanes of 50G PAM4. The cable adheres to the standardized OSFP form factor and complies rigorously with MultiSour-ce Agreement (MSA) specifications.

| Absolute Maximum Ratings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Unit | Min. | Max. | Notes |
| Supply Voltage | V | -0.3 | 3.6 |  |
| Data Input Voltage | V | -0.3 | 3.6 |  |
| Control Input Voltage | V | -0.3 | 3.6 |  |
| Operating Temperature | °C | 0 | 70 |  |
| Storage Temperature | °C | -40 | +85 |  |
| Relative Humidity (NonCondensing) | % | 5 | 85 |  |

| Operational Specification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Min | Typical | Max | Notes |
| Supply Voltage (Vcc) | V | 3.135 | 3.3 | 3.465 | Per End |
| Power Consumption | W |  |  | 1.5 | Per End |
| Operating Case Temperature | °C | 0 |  | 70 |  |
| Operating Relative Humidity | % | 0 |  | 85 |  |
| Modulation Format |  |  | 56G PAM4 |  |  |
| Bit Rate | Gbps | 8x50G to 8x50G |  |  |  |

| Electrical Characteristics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Min | Typical | Max | Notes |
| Characteristic Impedance | ohm | 90 | 100 | 110 |  |
| Time Propagation Delay (Informative) | ns | ….. | ….. | 4.9 |  |

| SI Performance |  |  |  |
| --- | --- | --- | --- |
| Item | Parameter | Require | Reference |
| 1 | ILdd Insertion loss at 13.28 GHz | 17.16 dB (Max.) | IEEE 802.3cd Section Section 136.11.2 |
| 2 | ILdd Insertion loss at 13.28 GHz | 8 dB (Min.) | IEEE 802.3cd Section Section 136.11.2 |
| 3 | ERL Minimum cable assembly | >11 dB*. | IEEE 802.3cd Section Section 136.11.3 |
| 4 | RLcd Differentialmode to commonmode return loss | 0.01GHz – 19GHz Equation (92–28) | IEEE 802.3cd Section 136.11.4 |
| 5 | ILcd Differentialmode to commonmode insertion loss | 0.01GHz – 19GHz Equation (92–29) | IEEE 802.3cd Section 136.11.5 |
| 6 | RLcc Commonmode to commonmode return loss | 0.01GHz – 19GHz Equation (92–30) | IEEE 802.3cd Section Section 136.11.6 |
| 7 | COM | 3dB (Min.) | IEEE 802.3ck Section 162.11.7 |
| *Cable assemblies with a com greater than 4 dB are not required to meet minimum ERL |  |  |  |

# Cable Wiring

OSFP 400G Active Redriver Cable Assembly Wiring

# Memory Map information (CMIS Version)

| Lower Memory Overview |  |  |  |
| --- | --- | --- | --- |
| Address | Size | Subject Area | Description |
| 0-3 | 4 | ID and Status Area | Module ID from SFF8024 list, version number, Type and status |
|  |  |  | Flat mem indication, CLEI present indicator, Maximum TWI speed, Current state of Module, Current state of the Interrupt signal |
| 4-7 | 4 | Lane Flag Summary | Flag summary of all lane flags on pages 10h1Fh |
| 8-13 | 6 | ModuleLevel Flags | All flags that are not lane or data path specific |
| 14-25 | 12 | ModuleLevel Monitors | Monitors that are not lane or data path spedific |
| 26-30 | 5 | Module Global Controls | Controls applicable to the module as a whole |
| 31-36 | 6 | ModuleLevel Flag Masks | Masking bits for the ModuleLevel flags |
| 37-38 | 2 | CDB Status Area | Status of most recent CDB command |

| 39-40 | 2 | Module Firmware Version | Module Firmware Version |
| --- | --- | --- | --- |
| 41-63 | 23 | Reserved Area | Reserved for future standardization |
| 64-82 | 19 | Custom Area | Vendor or module type specific use |
| 83-84 | 2 | Inactive Firmware Version | Version Number of Inactive Firmware. Values of 00h indicates module supports only a single image. |
| 85-117 | 33 | Application Advertising | Combinations of host and media interfaces that are supported by module data path(s） |
| 118-125 | 8 | Password Entry and Change |  |
| 126 | 1 | Bank Select Byte | Bank address of currently visible Page |
| 127 | 1 | Page Select Byte | Page address of currently visible Page |

| Page 00h Overview |  |  |  |
| --- | --- | --- | --- |
| Address | Size (bytes) | Name | Description |
| 128 | 1 | Identifier | Identifier Type of module |
| 129-144 | 16 | Vendor name | Vendor name (ASCII) |
| 145-147 | 3 | Vendor OUI | Vendor IEEE company ID |
| 148-163 | 16 | Vendor PN | Part number provided by vendor (ASCII) |
| 164-165 | 2 | Vendor rev | Revision level for part number provided by vendor (ASCII) |
| 166-181 | 16 | Vendor SN | Vendor Serial Number (ASCII) |
| 182-189 | 8 | Date Code |  |
| 190-199 | 10 | CLEI code | Common Language Equipment Identification code |
| 200-201 | 2 | Module power characteristics |  |
| 202 | 1 | Cable assembly length |  |
| 203 | 1 | Media Connector Type |  |
| 204-209 | 6 | Copper Cable Attenuation |  |
| 210-211 | 2 | Cable Assembly Lane Information |  |
| 212 | 1 | Media Interface Technology |  |
| 213-220 | 8 | Reserved |  |
| 221 | 1 | Custom |  |
| 222 | 1 | Checksum | Includes bytes 128221 |
| 223-255 | 33 | Custom Info NV |  |
| Note: For the above, refer to Common Management Interface Specification Rev5.0. |  |  |  |

# Mechanical Specifications

OSFP Form Factor

# OSFP PIN Function Definitions

| Pin | Logic | Symbol | Description |
| --- | --- | --- | --- |
| 1 |  | GND | Ground |
| 2 | CML-I | Tx2p | Transmitter Data Non-Inverted |
| 3 | CML-I | Tx2n | Transmitter Data Inverted |
| 4 |  | GND | Ground |
| 5 | CML-I | Tx4p | Transmitter Data Non-Inverted |
| 6 | CML-I | Tx4n | Transmitter Data Inverted |
| 7 |  | GND | Ground |
| 8 | CML-I | Tx6p | Transmitter Data Non-Inverted |
| 9 | CML-I | Tx6n | Transmitter Data Inverted |
| 10 |  | GND | Ground |
| 11 | CML-I | Tx8p | Transmitter Data Non-Inverted |
| 12 | CML-I | Tx8n | Transmitter Data Inverted |
| 13 |  | GND | Ground |
| 14 | LVCMOS-I/O | SCL | 2-wire serial interface clock |
| 15 |  | VCC | +3.3V Power |
| 16 |  | VCC | +3.3V Power |
| 17 | Multi-Level | LPWn/PRSn | Low-Power Mode / Module Present |
| 18 |  | GND | Ground |
| 19 | CML-O | Rx7n | Receiver Data Inverted |
| 20 | CML-O | Rx7p | Receiver Data Non-Inverted |
| 21 |  | GND | Ground |
| 22 | CML-O | Rx5n | Receiver Data Inverted |
| 23 | CML-O | Rx5p | Receiver Data Non-Inverted |

| 24 |  | GND | Ground |
| --- | --- | --- | --- |
| 25 | CML-O | Rx3n | Receiver Data Inverted |
| 26 | CML-O | Rx3p | Receiver Data Non-Inverted |
| 27 |  | GND | Ground |
| 28 | CML-O | Rx1n | Receiver Data Inverted |
| 29 | CML-O | Rx1p | Receiver Data Non-Inverted |
| 30 |  | GND | Ground |
| 31 |  | GND | Ground |
| 32 | CML-O | Rx2p | Receiver Data Non-Inverted |
| 33 | CML-O | Rx2n | Receiver Data Inverted |
| 34 |  | GND | Ground |
| 35 | CML-O | Rx4p | Receiver Data Non-Inverted |
| 36 | CML-O | Rx4n | Receiver Data Inverted |
| 37 |  | GND | Ground |
| 38 | CML-O | Rx6p | Receiver Data Non-Inverted |
| 39 | CML-O | Rx6n | Receiver Data Inverted |
| 40 |  | GND | Ground |
| 41 | CML-O | Rx8p | Receiver Data Non-Inverted |
| 42 | CML-O | Rx8n | Receiver Data Inverted |
| 43 |  | GND | Ground |
| 44 | Multi-Level | INT/RSTn | Module Interrupt / Module Reset |
| 45 |  | VCC | +3.3V Power |
| 46 |  | VCC | +3.3V Power |
| 47 | LVCMOS—I/O | SDA | 2-wire serial interface data |
| 48 |  | GND | Ground |

| 49 | CML-I | Tx7n | Transmitter Data Inverted |
| --- | --- | --- | --- |
| 50 | CML-I | Tx7p | Transmitter Data Non-Inverted |
| 51 |  | GND | Ground |
| 52 | CML-I | Tx5n | Transmitter Data Inverted |
| 53 | CML-I | Tx5p | Transmitter Data Non-Inverted |
| 54 |  | GND | Ground |
| 55 | CML-I | Tx3n | Transmitter Data Inverted |
| 56 | CML-I | Tx3p | Transmitter Data Non-Inverted |
| 57 |  | GND | Ground |
| 58 | CML-I | Tx1n | Transmitter Data Inverted |
| 59 | CML-I | Tx1p | Transmitter Data Non-Inverted |
| 60 |  | GND | Ground |

OSFP Electrical Pad Layout

For detail mechanical information, please refer to the related document of OSFP MSA.

LFOO-PU1xxxC

# Disclaimer

Images are for illustrative purposes only and may not accurately represent the actual

product. Specifications and appearances are subject to change without prior notice.

The information provided in this datasheet is subject to change without notice. Lumulus Technologies assumes no responsibility for any errors or omissions. Performance may vary depending on application and operating conditions. Product specifications are pro-vided for reference only and do not constitute a warranty.

For assistance and more information, please contact your sales representative or write

to sales@lumulus.com

# Lumulus Technologies, Inc. USA	 Headquarter

9685 Via Excelencia Suite 106

San Diego, CA 92126 USA

(+1) 858-397-2228

www.lumulus.com

sales@lumulus.com

Lumulus Technologies, Inc. (Thailand Branch)

# Production Site

30/30 Moo2, Tambol Wangchula Amphur, Wagnoi, Phranakorn

Sri Ayutthagya - 13170, Thailand

# Lumulus Tech offers 24/7 support,

- with experts ready to assist with all your technical needs and questions.

tech.support@lumulus.com	sales@lumulus.com

4

4

www.lumulus.com 9685 Via Excelencia, San Diego, CA 92126	Rev.A—17/01/25 www.lumulus.com 9685 Via Excelencia, San Diego, CA 92126	Rev.A—14/04/2026